# CNET600 Lab 01 – Introduction to Computer Networks - Exercises

Read or skim the references shown and complete the exercises below.

## Networking Theory:

## Readings

# CompTIA Network+ Certification Study Guide, Seventh Edition (Exam N10-007), 7th Edition

by Glen E. Clarke

ISBN: 9781260122053

https://www.oreilly.com/library/view/comptia-network-certification/9781260122053/

Read or Skim the following sections:

Computer Networking

- Chapter 8. Subnetting and Routing (Read up to Routing Concepts)

## Lab Exercise – Network Theory (20 mks)

Answer the following questions. Show your work and any reasoning used to calculate your results:

- Subnet the following network with 2 borrowed bits: 125.0.0.0/8

  - What class is the original network? (1 mk)

Ans. Class A. (IP addresses with a first octet between 1 and 127 belong to Class A.)

  - How many new networks (subnets) are created? (2 mks)

Ans. 4 subnets.

(Using formula 2n where n is the number of borrowed bits: 22 = 4)

  - What is the new subnet mask in dotted decimal and in “slash” or prefix notation? (2 mks)

Ans.  01. Dotted Decimal: 255.192.0.0. (128+64=192)

02. Slash or Prefix Notation: /10            (8+2)

(The original /8 mask is 11111111.00000000.00000000.00000000.

Borrowing 2 bits changes the second octet to 1100 0000 (which is 192 =128+64).

Total bits: 8 + 2 = 10.

New mask 255.192.0.0/10 (1111 1111.1100 0000.0000 0000. 0000 0000)

  - How many host addresses are available on each new subnet? (2 mks)

Ans. 4,194,302 hosts.

(2n - 2, where n is the number of remaining host bits.

Total bits (32) - network bits (10) = 22 host bits.

Number of hosts = 222- 2 = 4,194,304 - 2 = 4,194,302.)

  - What is the Block size? (1 mks)

Ans. Block Size: 64. (If we consider 10 bits on the network side, we will borrow 1st and 2nd HSB of 2nd octet. 2nd octet will be (1100 0000) (if we consider position of the bit, 2nd (from right) digit is the LSB that is 64 (192,64,0,0,0,0,0,0)

  - List out each new network with its range of available host addresses and show the broadcast address. (4 mks)

Ans.

| Subnet (with two bits) | Network IP | Host IP Range | Broadcast IP |
| --- | --- | --- | --- |
| 1 | 125.0.0.0 | 125.0.0.1 – 125.63.255.254 | 125.63.255.255 |
| 2 | 125.64.0.0 | 125.64.0.1 – 125.127.255.254 | 125.127.255.255 |
| 3 | 125.128.0.0 | 125.128.0.1 – 125.191.255.254 | 125.191.255.255 |
| 4 | 125.192.0.0 | 125.192.0.1 – 125.255.255.254 | 125.255.255.255 |

(With two bits we can create 2^2=4 subnet. Network IP will be the first address and broadcast IP will be the last address of the respective subnet.)

- If the network in A(1), 125.0.0.0/8 was instead subnetted with 6 borrowed bits

  - How many new networks would be created (2 mks)

Ans. 64 subnets. (26= 64.)

  - How many hosts would each network have? (2 mks)

Ans. 262,142 hosts. (New prefix is /14 (8 + 6). Host bits = 32 - 14 = 18.

218 - 2 = 262,144 - 2 = 262,142.)

- If the network in A(1), 125.0.0.0/8 was instead subnetted to create networks with 1022 hosts each

  - How many borrowed bits would be needed? (2 mks)

Ans. 14 borrowed bits. (To find host bits (n), we use 2n - 2 = 1022.

2n= 1024.

2n=210

So, n =10. So, 10 bits are needed for hosts.

Remaining bits for the network = 32 (total bits) – 10 (host bits) = 22 (network bits).

Borrowed bits = 22 (total network bits) - 8 (default bits) = 14 borrowed bits.

  - How many networks (subnets) would be created? (2 mks)

Ans. 16,384 subnets. (214 = 16,384).